CALLER-1	Characteristics

General technical information

Electrical

-         Power supply: 12 VDC +/- 5% 1,2 A

-         720 mA average in GSM 900 at Tx power max 2W

-         920 mA average in GSM/GRPS 900 at Tx power max 2W

-         100 mA in idle mode

Physical

-         Absolute maximum dimension: 165 x 127 x 30 mm

-         Weight: 600 g

-         Casing: Complete shielding-stainless steel

-         Mounting: 4 screw holes

-         Operating temperature range: -5°C to + 55°C

Optimal storage temperature: -35°C to +85°C

Analog Telephone/Analog PBX interface

-         Tip/Ring Voltage –48V

-         Line current 25ma

-         Ringing Voltage 140 Vpp

Load   3REN

Crest Factor        1.25

Frequency          20 Hz

Cadency             2s/4s

-         Answer supervision - Reverse polarity

-         DTMF compliancy

-         Dial Tone 400 Hz

Busy Tone 400 Hz, cadency 500ms/500ms

GSM circuit Data Features

Data circuit asynchronous, transparent and non transparent up to 14,400 b/s

Automatic fax group 3 (Class 1 & 2)

Alternate speech and fax

MNP2, V.42bis

## Telephone Line Characteristics

### DC Characteristics

Ringing Voltage:	-70 V DC typical

On hook voltage:	-58 V DC typical

Off hook Voltage:	-24 V DC typical

Off hook Loop current:	27mA @ 600 Ohm programming resistive

Input Impedance:	480 Ohms to 720 Ohms,  600 ohm typical

### DTMF Dialling:

Detection level:	-25dBm to +3dBm

On time:	50msec.

Off time:	12 characters

### Ringing Signal:

Feeding type:		Balanced signal

Distortion:		Less than 5%

Ring drive capability		3 type A

Minimum ring Voltage:		40 Vrms – crest factor 1.2

Frequency:		(20 Hz )

### Normal dialling tone:

### ( can be changed per customer request )

Frequency:	  400Hz

Level:		2dB

Cadence:			          continuous

### Congestion / Busy Tone: ( can be changed per customer request )

Frequency:		400Hz

Level:	   2dB

Cadence:		500ms 20% signal

500ms 20% pause

## Environmental characteristics

Compliance:

The CALLER-1 complies to the following requirements:

## Temperature     -10 to 70

Humidity            5% to 95%

## Packing

Per ElectrOnde standard

## Transportation

Transport conditions are according to ElectrOnde Standards.

## Storage Condition

Storage conditions are according to class 1.1, ETS 300 019 –1-1 (-20°C +80°C)

-25°C to +65°C

## Mounting

### Wall Mounting

In case you wish to mount on wall see instructions above in General Information.

configuration the CALLER-1 attached to the wall with one screw.

### Desk – Top Mount

In desk top mounting configuration on four rubber standoffs, it should be mounted

at least .5 meter from the telephone set.

### Packing

Each CALLER-1 package contains:

One CALLER-1 unit, metal box.

One Omni antenna or an external antenna (optional)

This booklet which includes a user manual.

### Labels

Each CALLER-1  labelled as the following :

Product ID: located at the box side( in future units).

The label contains product model number, Serial number, Date and code identified

as WWYY

(1’st two digits identify the chronological week in the year,  2nd tow digits identify the

year), Hardware and Software

version.        For Example: Logo, Model No, Serial No, Rev.

HardwareX/SoftwareY:0X/0Y , Date: Month/Year

## Manufacturing

At ElectrOnde we bear in mind at all times manufacturing at high Quality and cost effective are the most significant considerations in the design for Manufacturing of the CALLER-1.

## Shipment

All goods will be sold FOB, customer will pay for delivery from factory.

Customer will advise on the way the products will be shipped.

Customer will take care of shipment including all necessary payments for

insurance, delivery, custom, etc.  as FOB requires.

## Reliability

The CALLER-1 Field Quality goal is and will be determined by reliability prediction actual ALT results.

Our target is:      0.5-1.0% per month, this goal is susceptible to changes.